Change Request

for the update of ISO 20022 financial repository items

Network Configuration

- Origin of the request:

A.1 Submitter:

nexo A. I. S. B. L. (formerly EPASOrg).

A.2 Contact person:

Louis JENNY, louis-jenny@nexo-standards.org, +33 610 277 004

A.3 Sponsors:

Not applicable.

- Related messages:

  - AcceptorAuthorisationRequest (caaa.001.001.04)

  - AcceptorCompletionAdvice (caaa.003.001.04)

  - AcceptorCancellationRequest (caaa.005.001.04)

  - AcceptorCancellationAdvice (caaa.007.001.04)

  - AcceptorReconciliationRequest (caaa.009.001.04)

  - AcceptorBatchTransfer (caaa.011.001.04)

  - AcceptorDiagnosticRequest (caaa.013.001.04)

  - AcceptorDiagnosticResponse (caaa.014.001.04)

  - AcceptorCurrencyConversionRequest (caaa.016.001.02)

  - StatusReport  (catm.001.001.04)

  - ManagementPlanReplacement (catm.002.001.04)

  - AcceptorConfigurationUpdate (catm.003.001.04)

  - MaintenanceDelegationRequest (catm.005.001.01)

- Description of the change request:

Add in the host addresses of the ManagementPlanReplacement, AcceptorConfigurationUpdate and TerminalManagementRejection TMS messages, the following optional elements:

- The local proxy configuration,

- The service provider configuration,

- The configuration of the gateway between a mobile network and a packet network.

Add in the acquirer information (message component Acquirer4) of the AcceptorAuthorisationRequest, AcceptorCompletionAdvice, AcceptorCancellationRequest, AcceptorCancellationAdvice, AcceptorReconciliationRequest, AcceptorBatchTransfer, AcceptorDiagnosticRequest and AcceptorDiagnosticResponse messages, the optional telecommunication address of the acquirer or of the intermediary agent.

- Purpose of the change:

The security rules at the card acceptor’s side can prevent the POI from connecting directly to an IP network without going through a local proxy. This local configuration may be configured remotely.

In some cases, for instance GPRS on a private gateway), the POI only has access to the acquirer or intermediary agent through an IP address provided by a service provider.

The IP address of the service provider depends on the network used by the POI. Then, if the POI can use several types of network to reach the acquirer (Ethernet, GPRS), the IP address of the service provider can be different according to the typology of the network (public IP address on a public network and private IP address on a private network).

Then for each kind of network available on the POI, the service provider should be configurable.

- Urgency of the request:

Publication of new message versions in April/May of 2016.

- Business examples:

Tier 1 retailers centralise the local communication configuration of their stores.

Acquirers use private telecommunication service providers to standardise their network access and reduce communication costs.

- SEG recommendation:

| Consider |  | X | Timing |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | - Next yearly cycle: 2015/2016 (the change will be considered for implementation in the yearly maintenance cycle which starts in 2015 and completes with the publication of new message versions in the spring of 2016) |  | X |  |
|  |  |  | - At the occasion of the next maintenance of the messages (the change will be considered for implementation, but does not justify maintenance of the messages in its own right – will be pending until more critical change requests are received for the messages) |  |  |  |
|  |  |  | - Urgent unscheduled (the change justifies an urgent implementation outside of the normal yearly cycle) |  |  |  |
|  |  |  | - Other timing: |  |  |  |

Comments:

Card SEG Conference Call on the 23rd of June 2015:

Participants:

- Cervenka, Karen

- DuVall Richard A.

- Gygax, Martin (Apologised, endorsement given by mail)

- Herwig, Reinhard

- Jenny, Louis

- Lafeuillade, Laurent (Apologised)

- Powell, Glenn

- Sasor, Tomasz

- Thiemann, Alan

- Vanobberghen, William

The participants agree with the concept of the message enhancements described in the CR above.

| Reject |  |
| --- | --- |

Reason for rejection: